Supporting Information for

A numerical model study of the main factors contributing to hypoxia and its sub-seasonal to interannual variability off the Changjiang Estuary

# Haiyan Zhang1, 2, Katja Fennel1, Arnaud Laurent1, Changwei Bian3

1Department of Oceanography, Dalhousie University, Halifax, Nova Scotia, Canada

2School of Marine Science and Technology, Tianjin University, Tianjin, China

3 Physical Oceanography Laboratory/CIMST, Ocean University of China, and Qingdao National Laboratory for Marine Science and Technology, Qingdao, China

Submitted to Biogeosciences

Model-data comparisons of temperature and salinity

Figure S1. Monthly averaged (2008-2013) SST in the model (top row) and from NOAA AVHRR (row below) in February, May, August and November. 2-dimensional histograms (bottom row) show the corresponding comparisons between model SST and AVHRR SST with correlation reported in each panel. The 1-to-1 line is shown in black. The color scale of the bottom row indicates the number of data pairs in each bin.

Figure S2. Simulated surface salinity (map) compared with observations (dots) during nine cruises from 2011 to 2013.

Figure S3. The same as Figure S2 except for bottom salinity.

Figure S4. The same as Figure S2 except for surface temperature.

Figure S5. The same as Figure S2 except for bottom temperature.

Assessment of the dominant currents

Figure S6. Simulated monthly mean surface currents in February and August. Black arrows represent currents with velocity >= 0.4 m/s, while blue arrows denote velocity <0.4 cm/s.

Model-data comparisons of surface chlorophyll

Figure S7. The same as Figure S1 except for surface chlorophyll. Observed surface chlorophyll is from MODIS Terra. No observations are available for the white areas near the coast.

Figure SX. Wind stress (black), mean bottom oxygen in the northern and southern zones (dark and light blue), and total hypoxic extent (orange) and FW plume extent (purple) throughout July to October from 2008 to 2013. The filled and open circles indicate a variables’ value at the beginning and after high-wind events. High-wind days/events are indicated by the dark/light gray shading.

Biogeochemical model parameters

| Symbol | Parameter | Value | Units |
| --- | --- | --- | --- |
| μ0 | phytoplankton growth rate at 0℃ | 0.59 | d-1 |
| kNO3 | half-saturation concentration for uptake of NO3 | 0.5 | mmol N m-3 |
| kNH4 | half-saturation concentration for uptake of NH4 | 0.5 | mmol N m-3 |
| kPO4 | half-saturation concentration for uptake of PO4 | 0.03 | mmol P m-3 |
| α | initial slope of the P-I curve | 0.025 | mgC mgChl-1 (W m-2)-1 d-1 |
| kI | light intensity at which the inhibition of nitrification is half-saturated | 0.1 | W m-2 |
| I0 | threshold for light-inhibition of nitrification | 0.0095 | W m-2 |
| mP | phytoplankton mortality | 0.15 | d-1 |
| τ | aggregation parameter | 0.04 | (mmol N m-3)-1 d-1 |
| Θmax | maximum chlorophyll to phytoplankton ratio | 0.053 | mgChl mgC-1 |
| gmax | maximum grazing rate | 0.6 | d-1 |
| kP | half-saturation concentration of phytoplankton ingestion | 2 | (mmol N m-3)2 |
| β | assimilation efficiency | 0.75 | dimensionless |
| lBM | excretion rate due to basal metabolism | 0.1 | d-1 |
| lE | maximum rate of assimilation related excretion | 0.1 | d-1 |
| mZ | zooplankton mortality | 0.1 | (mmol N m-3)-1 d-1 |
| rSD | remineralization rate of suspended detritus | 0.3 | d-1 |
| rLD | remineralization rate of large detritus | 0.01 | d-1 |
| rRD | remineralization rate of riverine dissolved organic matter | 0.03 | d-1 |
| nmax | maximum nitrification rate | 0.2 | d-1 |
| wPhy | sinking velocity of phytoplankton | 0.1 | m d-1 |
| wSDet | sinking velocity of suspended detritus | 0.1 | m d-1 |
| wLDet | sinking velocity of larger particles | 5 | m d-1 |

Table S1 Biological model parameters used in this study.

Oxygen budget

| Water column | year | northern hypoxic region |  |  |  |  |  | southern hypoxic region |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | air-sea | adv | pp | WR | SOC | sum | air-sea | adv | pp | WR | SOC | sum |
| Whole column | 2008 | -2.75 | -2.35 | 9.03 | -2.98 | -3.24 | -2.28 | -1.36 | -1.22 | 4.81 | -2.13 | -1.78 | -1.68 |
|  | 2009 | -2.70 | -1.94 | 9.58 | -3.32 | -3.40 | -1.79 | -1.22 | -1.07 | 5.10 | -2.39 | -1.80 | -1.38 |
|  | 2010 | -3.13 | -1.75 | 9.04 | -2.98 | -3.38 | -2.20 | -1.88 | -1.00 | 5.73 | -2.44 | -2.19 | -1.79 |
|  | 2011 | -2.78 | -2.51 | 8.86 | -2.99 | -2.97 | -2.40 | -1.24 | -1.40 | 4.64 | -2.14 | -1.68 | -1.82 |
|  | 2012 | -2.78 | -1.80 | 8.56 | -2.64 | -3.08 | -1.74 | -1.19 | -1.02 | 4.80 | -2.08 | -1.75 | -1.24 |
|  | 2013 | -2.57 | -1.86 | 9.59 | -3.03 | -3.26 | -1.13 | -1.46 | -1.47 | 5.77 | -2.23 | -1.90 | -1.29 |
|  |  | diff | adv | pp | WR | SOC | sum | diff | adv | pp | WR | SOC | sum |
| Bottom water | 2008 | 2.96 | 1.81 | 0.17 | -0.78 | -3.24 | 1.11 | 2.81 | 0.13 | -0.48 | -1.78 | 1.78 | 1.11 |
|  | 2009 | 2.96 | 2.47 | 0.15 | -0.84 | -3.40 | 1.06 | 2.05 | 0.12 | -0.53 | -1.80 | 0.90 | 1.06 |
|  | 2010 | 2.71 | 1.39 | 0.12 | -0.75 | -3.38 | 0.98 | 1.39 | 0.10 | -0.57 | -2.19 | -0.29 | 0.98 |
|  | 2011 | 3.03 | 1.21 | 0.19 | -0.86 | -2.97 | 1.10 | 1.79 | 0.14 | -0.53 | -1.68 | 0.82 | 1.10 |
|  | 2012 | 2.53 | 0.96 | 0.12 | -0.69 | -3.08 | 1.18 | 2.12 | 0.11 | -0.51 | -1.75 | 1.15 | 1.18 |
|  | 2013 | 3.44 | 2.73 | 0.20 | -0.84 | -3.26 | 1.06 | 3.34 | 0.13 | -0.52 | -1.90 | 2.10 | 1.06 |

Table S2. Oxygen budget for the period during which oxygen decreases (March to August) in different years for the whole water column and the bottom water respectively (unit: mol O2 m-2).